| CỤC VIỄN THÔNG TRUNG TÂM KIỂM ĐỊNH VÀ CHỨNG NHẬN 2 VILAS 197 ------------- | CỘNG HÒA XÃ HỘI CHỦ NGHĨA VIỆT NAM Độc lập - Tự do - Hạnh phúc ------------------------- |
| --- | --- |
| Số: 0276/2014/PTN-SP | Tp Hồ Chí Minh, ngày 21 tháng 04 năm 2014 |

KẾT QUẢ ĐO KIỂM

MÁY ĐIỆN THOẠI DI ĐỘNG

- Đơn vị đề nghị đo kiểm:

-	Tên đơn vị: CÔNG TY TNHH QUỐC TẾ HUÂN BẰNG

-	Địa chỉ: 125/208 Lương Thế Vinh, Phường Tân Thới Hòa, Quận Tân Phú, TP.HCM

-	ĐT: (08) 39618160                                  Fax: (08) 39618163

2.   Sản phẩm được đo kiểm:

- 	Tên sản phẩm: Điện thoại di động.

-   Ký hiệu: Dopod P3500

- IMEI 1: 355807044280815

- IMEI 2: 355807044380722

- Hãng sản xuất: SHENZHEN JIXIANGTIAN IMP&EXP CO., LTD

- Nước sản xuất: Trung Quốc

(Các thông tin trên được ghi theo phiếu yêu cầu đo kiểm ngày 11/04/2014 của đơn vị đề nghị đo kiểm)

- Tình trạng mẫu: Mẫu do khách hàng mang đến Phòng Thử nghiệm.

3. 	Địa điểm đo kiểm:

- Phòng Thử nghiệm (VILAS 197).

- Địa chỉ: 60 Tân Canh, Phường 01, Quận Tân Bình, TP. HCM.

4.	Nhóm đo kiểm:

- Nguyễn Kiều Diễm;

- Phạm Minh Trang.

5.	Thời gian đo kiểm:

- 	Ngày 21 tháng  04 năm 2014

6.	Thiết bị đo kiểm:

-   CMU 200 – Universal Radio Communication Tester

7.	Điều kiện môi trường:

- 	Nhiệt độ:  25C

-	Độ ẩm:     50%

8.	Tiêu chuẩn đánh giá:

- 	QCVN 12:2010/BTTTT-"Quy chuẩn kỹ thuật quốc gia về máy di động GSM pha 2 và 2+) "

9.    Nội dung đo kiểm:

| STT | Chỉ tiêu đo kiểm | Cơ quan công nhận / chỉ định |
| --- | --- | --- |
| 1 | Công suất trung bình | Bộ Thông tin Truyền thông Văn phòng Công nhận chất lượng |
| 2 | Định thời cụm | Bộ Thông tin Truyền thông Văn phòng Công nhận chất lượng |
| 3 | Sai số thời gian | Bộ Thông tin Truyền thông Văn phòng Công nhận chất lượng |
| 4 | Sai số pha | Bộ Thông tin Truyền thông Văn phòng Công nhận chất lượng |
| 5 | Sai số tần số | Bộ Thông tin Truyền thông Văn phòng Công nhận chất lượng |
| 6 | Phổ tín hiệu đầu ra | Bộ Thông tin Truyền thông Văn phòng Công nhận chất lượng |
| 7 | RX level | Bộ Thông tin Truyền thông Văn phòng Công nhận chất lượng |
| 8 | BER | Bộ Thông tin Truyền thông Văn phòng Công nhận chất lượng |

10.	Kết quả đo kiểm:

- Kết quả đo kiểm chi tiết trong phụ lục kèm theo.

- Kết quả đo kiểm chỉ có giá trị với mẫu đo kiểm.

| ĐẠI DIỆN NHÓM ĐO KIỂM NGUYỄN KIỀU DIỄM | QUẢN LÝ KỸ THUẬT NGUYỄN PHÚ CƯỜNG |
| --- | --- |

# PHỤ LỤC

(Kèm theo kết quả đo kiểm số 0276/2014/PTN-SP)

# SIM-1

Date & Time:	Monday, April 14, 2014  14:28:51 PM

Operator:	noname

CMU Ident:	Rohde&Schwarz,CMU 200-1100.0008.02,121644,V5.20

Options:	B11/B12,B21Var14,B54Var14,B56Var54,B66,COPROC_FULL,B83,B89,B95,WDDC400,K16,K17,K20,K21,K22

K23,K24,K27,K28,K29,K42,K43,K56,K57,K58,K59,K60,K61,K62,K63,K64,K65,K66,K67,K68,K69,K83,K84

K85,K86,K88,K839,K849,K859,K869,FMR7,Intel Celeron M,480 MB

rxtx1

Sequence:	- - -

Test Name and Condition	Lower Limit	Upper Limit	Measured Value	Unit	P/F

Network: GSM 900, BCCH Channel/Level: 30/-85.0 dBm, TCH Channel: 62, TCH Level (dBm): -85.0, Support GSM, PCL: 15

(Main) Timeslot: 3, Attenuation (In/Out): 15.0/15.0 dB, Slot Mode: Single Slot, Bitstream: PR9, AuxTX: Off

Traffic Mode: Full Rate V1

IMSI:   1. 1.0123456789,Serial: 35580704-428081-5, Revision: PH2P, Number: -, Traffic: NAN, Power Class: 4

Call to Mobile:					Passed

GSM 900, Channel 72, PCL 5, Timeslot: 3, BS RF Level -102.0 dBm, Attenuation (In/Out) 17.0 dB / 17.0 dB

Average Power: 20 Bursts	31.00	35.00	33.05	dBm	Passed

Power Time Template: 20 Bursts					Passed

Timing Error: 20 Bursts	-2.00	2.00	-1.08	Bit	Passed

Phase Error Peak: 20 Bursts	-20.00	20.00	3.18	°	Passed

Phase Error RMS: 20 Bursts		5.00	0.96	°	Passed

Frequency Error: 20 Bursts	-90.00	90.00	-13.17	Hz	Passed

Spectrum due to Switching: 20 Bursts					Passed

Spectrum due to Modulation: 50 Bursts					Passed

RX Lev:	6	11	11		Passed

BBB (Fast BER)   88 Frames		0.20	0.00	%	Passed

GSM 900, Channel 20, PCL 5, Timeslot: 3, BS RF Level -102.0 dBm, Attenuation (In/Out) 17.0 dB / 17.0 dB

Average Power: 20 Bursts	31.00	35.00	31.40	dBm	Passed

Power Time Template: 20 Bursts					Passed

Timing Error: 20 Bursts	-2.00	2.00	-0.99	Bit	Passed

Phase Error Peak: 20 Bursts	-20.00	20.00	2.97	°	Passed

Phase Error RMS: 20 Bursts		5.00	1.01	°	Passed

Frequency Error: 20 Bursts	-90.00	90.00	-23.05	Hz	Passed

Spectrum due to Switching: 20 Bursts					Passed

Spectrum due to Modulation: 50 Bursts					Passed

RX Lev:	6	11	11		Passed

BBB (Fast BER)   88 Frames		0.20	0.00	%	Passed

GSM 900, Channel 110, PCL 5, Timeslot: 3, BS RF Level -102.0 dBm, Attenuation (In/Out) 17.0 dB / 17.0 dB

Average Power: 20 Bursts	31.00	35.00	34.06	dBm	Passed

Power Time Template: 20 Bursts					Passed

Timing Error: 20 Bursts	-2.00	2.00	-1.23	Bit	Passed

Phase Error Peak: 20 Bursts	-20.00	20.00	4.10	°	Passed

Phase Error RMS: 20 Bursts		5.00	1.07	°	Passed

Frequency Error: 20 Bursts	-90.00	90.00	-10.14	Hz	Passed

Spectrum due to Switching: 20 Bursts					Passed

Spectrum due to Modulation: 50 Bursts					Passed

RX Lev:	6	11	10		Passed

BBB (Fast BER)   88 Frames		0.20	0.15	%	Passed

GSM 900, Channel 72, PCL 10, Timeslot: 3, BS RF Level -102.0 dBm, Attenuation (In/Out) 17.0 dB / 17.0 dB

Average Power: 20 Bursts	20.00	26.00	25.18	dBm	Passed

Power Time Template: 20 Bursts					Passed

Timing Error: 20 Bursts	-2.00	2.00	-1.25	Bit	Passed

Phase Error Peak: 20 Bursts	-20.00	20.00	3.68	°	Passed

Phase Error RMS: 20 Bursts		5.00	1.04	°	Passed

Frequency Error: 20 Bursts	-90.00	90.00	-18.02	Hz	Passed

GSM 900, Channel 72, PCL 15, Timeslot: 3, BS RF Level -102.0 dBm, Attenuation (In/Out) 17.0 dB / 17.0 dB

Average Power: 20 Bursts	10.00	16.00	14.16	dBm	Passed

Power Time Template: 20 Bursts					Passed

Timing Error: 20 Bursts	-2.00	2.00	-1.24	Bit	Passed

Phase Error Peak: 20 Bursts	-20.00	20.00	3.13	°	Passed

Phase Error RMS: 20 Bursts		5.00	0.96	°	Passed

Frequency Error: 20 Bursts	-90.00	90.00	-16.79	Hz	Passed

Call Release Test:					Passed

MS Serial Number: 35580704-428081-5

Result:	44 Tests passed / 0 Tests failed                                    (Execution Time: 60.7 Seconds)

SIM-2

Date & Time:	Monday, April 14, 2014  14:33:50 PM

Operator:	noname

CMU Ident:	Rohde&Schwarz,CMU 200-1100.0008.02,121644,V5.20

Options:	B11/B12,B21Var14,B54Var14,B56Var54,B66,COPROC_FULL,B83,B89,B95,WDDC400,K16,K17,K20,K21,K22

K23,K24,K27,K28,K29,K42,K43,K56,K57,K58,K59,K60,K61,K62,K63,K64,K65,K66,K67,K68,K69,K83,K84

K85,K86,K88,K839,K849,K859,K869,FMR7,Intel Celeron M,480 MB

rxtx1

Sequence:	- - -

Test Name and Condition	Lower Limit	Upper Limit	Measured Value	Unit	P/F

Network: GSM 900, BCCH Channel/Level: 30/-85.0 dBm, TCH Channel: 62, TCH Level (dBm): -85.0, Support GSM, PCL: 15

(Main) Timeslot: 3, Attenuation (In/Out): 15.0/15.0 dB, Slot Mode: Single Slot, Bitstream: PR9, AuxTX: Off

Traffic Mode: Full Rate V1

IMSI:   1. 1.0123456789,Serial: 35580704-438072-2, Revision: PH2P, Number: -, Traffic: NAN, Power Class: 4

Call to Mobile:					Passed

GSM 900, Channel 72, PCL 5, Timeslot: 3, BS RF Level -102.0 dBm, Attenuation (In/Out) 18.0 dB / 18.0 dB

Average Power: 20 Bursts	31.00	35.00	33.45	dBm	Passed

Power Time Template: 20 Bursts					Passed

Timing Error: 20 Bursts	-2.00	2.00	-1.29	Bit	Passed

Phase Error Peak: 20 Bursts	-20.00	20.00	3.23	°	Passed

Phase Error RMS: 20 Bursts		5.00	0.94	°	Passed

Frequency Error: 20 Bursts	-90.00	90.00	-18.79	Hz	Passed

Spectrum due to Switching: 20 Bursts					Passed

Spectrum due to Modulation: 50 Bursts					Passed

RX Lev:	6	11	11		Passed

BBB (Fast BER)   88 Frames		0.20	0.00	%	Passed

GSM 900, Channel 20, PCL 5, Timeslot: 3, BS RF Level -102.0 dBm, Attenuation (In/Out) 18.0 dB / 18.0 dB

Average Power: 20 Bursts	31.00	35.00	31.68	dBm	Passed

Power Time Template: 20 Bursts					Passed

Timing Error: 20 Bursts	-2.00	2.00	-1.22	Bit	Passed

Phase Error Peak: 20 Bursts	-20.00	20.00	3.10	°	Passed

Phase Error RMS: 20 Bursts		5.00	1.01	°	Passed

Frequency Error: 20 Bursts	-90.00	90.00	-25.44	Hz	Passed

Spectrum due to Switching: 20 Bursts					Passed

Spectrum due to Modulation: 50 Bursts					Passed

RX Lev:	6	11	11		Passed

BBB (Fast BER)   88 Frames		0.20	0.00	%	Passed

GSM 900, Channel 110, PCL 5, Timeslot: 3, BS RF Level -102.0 dBm, Attenuation (In/Out) 18.0 dB / 18.0 dB

Average Power: 20 Bursts	31.00	35.00	34.56	dBm	Passed

Power Time Template: 20 Bursts					Passed

Timing Error: 20 Bursts	-2.00	2.00	-1.36	Bit	Passed

Phase Error Peak: 20 Bursts	-20.00	20.00	3.99	°	Passed

Phase Error RMS: 20 Bursts		5.00	0.96	°	Passed

Frequency Error: 20 Bursts	-90.00	90.00	-13.88	Hz	Passed

Spectrum due to Switching: 20 Bursts					Passed

Spectrum due to Modulation: 50 Bursts					Passed

RX Lev:	6	11	11		Passed

BBB (Fast BER)   88 Frames		0.20	0.15	%	Passed

GSM 900, Channel 72, PCL 10, Timeslot: 3, BS RF Level -102.0 dBm, Attenuation (In/Out) 18.0 dB / 18.0 dB

Average Power: 20 Bursts	20.00	26.00	25.59	dBm	Passed

Power Time Template: 20 Bursts					Passed

Timing Error: 20 Bursts	-2.00	2.00	-1.35	Bit	Passed

Phase Error Peak: 20 Bursts	-20.00	20.00	3.45	°	Passed

Phase Error RMS: 20 Bursts		5.00	0.97	°	Passed

Frequency Error: 20 Bursts	-90.00	90.00	-19.69	Hz	Passed

GSM 900, Channel 72, PCL 15, Timeslot: 3, BS RF Level -102.0 dBm, Attenuation (In/Out) 18.0 dB / 18.0 dB

Average Power: 20 Bursts	10.00	16.00	14.60	dBm	Passed

Power Time Template: 20 Bursts					Passed

Timing Error: 20 Bursts	-2.00	2.00	-1.35	Bit	Passed

Phase Error Peak: 20 Bursts	-20.00	20.00	3.13	°	Passed

Phase Error RMS: 20 Bursts		5.00	1.01	°	Passed

Frequency Error: 20 Bursts	-90.00	90.00	-20.21	Hz	Passed

Call Release Test:					Passed

MS Serial Number: 35580704-438072-2

Result:	44 Tests passed / 0 Tests failed                               (Execution Time: 53.6 Seconds)

Hình 1: Mặt trước của máy, nắp và pin.

Hình 2: Mặt sau của máy, nắp và pin.

Hình 3: Ký hiệu, số IMEI của máy.